ANCILLARY VARIABLES IN PREPBUFR FILE (INCLUDING SENSIBLE WEATHER ELEMENTS)

(Last Update: 05/06/2010)

| Variable (units – or link if Code/Flag Table) (BUFR mnemonic) | SYNOP (BUFR type 000, subtypes 000-002) | METAR (BUFR type 000, subtype 007) | SHIP (BUFR type 001, subtype 001) | DRIFT-ING BUOY (BUFR type 001, subtype 002) | FIXED BUOY (BUFR type 001, subtype 003) | C-MAN (BUFR type 001, subtype 004) | MESONET (BUFR type 255, all subtypes) | RAOB/PIBAL (BUFR type 002, subtypes 001-003, 005, 009) | RECCO (BUFR type 004, subtype 005) | PIREP (BUFR type 004, subtype 002) | AMDAR (BUFR type 004, subtype 003) | E-ADAS/Canadian AMDAR (BUFR type 004, subtypes 006, 009) | TAMDAR (BUFR type 004, subtype 008) | MDCRS-ACARS (BUFR type 004, subtype 004) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADPSFC |  | SFCSHP |  |  |  | MESONET | ADPUPA |  | AIRCFT |  |  |  | AIRCAR |
| Duration in time over which total precip./total water equivalent measured (hrs) (.DTHTOPC) | yes |  | yes |  | yes | yes | yes4 |  |  |  |  |  |  |  |
| Total precip./total water equivalent (kg/m**2) (TOPC) | yes | yes5 | yes |  | yes | yes | yes |  |  |  |  |  |  |  |
| Present weather (PRWE) | YES | YES | YES |  | yes | yes |  |  | YES | yes |  |  |  |  |
| Vertical significance (surface observations) (VSSO) | YES | YES | YES |  |  |  |  |  |  |  |  |  |  |  |
| Cloud amount (CLAM) | YES | YES | YES |  |  |  |  | YES2 | yes | YES |  |  |  |  |
| Cloud type (CLTP) | YES | YES | YES |  |  |  |  | YES2 | yes | yes |  |  |  |  |
| Height of base of cloud (m) (HOCB) | yes6 | YES | yes |  |  |  |  | yes6,7 | yes | YES |  |  |  |  |
| Height of top of cloud (m) (HOCT) | yes |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Duration in time over which maximum temperature measured (hrs) (.DTHMXTM) |  | YES1 | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Maximum temperature (K) (MXTM) | YES | YES1 | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Duration of time over which minimum temperature measured (hrs) (.DTHMITM) | YES | YES1 | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Minimum temperature (K) (MITM) | YES | YES1 | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Direction of swell waves (deg.) (DOSW) | yes (not in subtype 002) |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Height of swell waves (m) (HOSW) | yes (not in subtype 002) |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Period of swell waves (sec.) (POSW) | yes (not in subtype 002) |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Restrictions on redistribution (RSRD) | yes (only in subtype 000) |  |  |  |  |  | YES |  |  |  | YES | YES |  | YES (also YES in subtype 007) |
| Expiration of restrictions on redistribution (hrs) (EXPRSRD) | yes (only in subtype 000) |  |  |  |  |  | yes |  |  |  | YES | YES |  | YES (also YES in subtype 007) |
| Altimeter setting (Pa) (ALSE) |  | YES |  |  |  |  | YES |  |  |  |  |  |  |  |
| Sea surface temperature (K) (SST1) | yes | yes | YES | YES3 | YES | YES (also YES in tide gauge subtype 005) |  | yes (subtype 003) yes (all others, incl. drops?) | yes |  |  |  |  |  |
| Depth below sea surface (for SST1 measurement) (m) (DBSS) |  |  |  | yes3 |  |  |  |  |  |  |  |  |  |  |
| Method of sea surface temperature measurement (MSST) | yes |  | YES |  | YES | YES |  |  |  |  |  |  |  |  |
| Relationship to horizontal visibility (.REHOVI) |  | YES |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal visibility (m) (HOVI) | YES | YES | YES |  | YES | yes |  |  |  |  |  |  |  |  |
| Vertical visibility (m) (VTVI) | yes | yes | yes |  |  |  |  |  |  |  |  |  |  |  |
| Past weather 1 (PSW1) | YES |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Past weather 2 (PSW2) | YES |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Peak wind speed (m/s) (PKWDSP) |  | yes |  |  | YES | YES |  |  |  |  |  |  |  |  |
| Peak wind direction (deg.) (PKWDDR) |  | yes |  |  | YES | YES |  |  |  |  |  |  |  |  |
| Duration in time over which maximum wind speed (gust) measured (min) (.DTMMXGS) | yes | YES | yes |  | YES | YES |  |  |  |  |  |  |  |  |
| Maximum wind speed (gust) (m/s) (MXGS) | YES | YES | yes |  | YES | YES | YES |  |  |  |  |  |  |  |
| Maximum wind gust direction (deg.) (MXGD) |  |  |  |  |  |  | yes |  |  |  |  |  |  |  |
| Total precipitation past 1-hour (kg/m**2) (TP01) | yes | YES | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Total precipitation past 3-hours (kg/m**2) (TP03) | yes | yes | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Total precipitation past 6-hours (kg/m**2) (TP06) | YES | YES | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Total precipitation past 12-hours (kg/m**2) (TP12) | yes |  | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Total precipitation past 24-hours (kg/m**2) (TP24) | YES | yes | yes |  | yes | yes |  |  |  |  |  |  |  |  |
| Total sunshine (min.) (TOSS) |  | yes |  |  |  |  |  |  |  |  |  |  |  |  |
| Total cloud cover (%) (TOCC) | YES |  | YES |  |  |  |  |  |  |  |  |  |  |  |
| Height above surface of base of lowest cloud seen (HBLCS) | YES6 |  | YES |  |  |  |  | YES2,6,7 |  |  |  |  |  |  |
| 10 meter extrapolated wind speed (m/s) (XS10) |  |  |  |  | YES | YES |  |  |  |  |  |  |  |  |
| 20 meter extrapolated wind speed (m/s) (XS20) |  |  |  |  | YES | YES |  |  |  |  |  |  |  |  |
| Surface wind direction (deg.) (WDIR1) |  |  |  |  |  |  |  |  | yes |  |  |  |  |  |
| Surface wind speed (m/s) (WSPD1) |  |  |  |  |  |  |  |  | yes |  |  |  |  |  |
| Duration of time over which depth of fresh snow measured (hrs) (.DTHDOFS) | yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Depth of fresh snow (m) (DOFS) | yes | yes |  |  |  |  |  |  |  |  |  |  |  |  |
| Total snow depth (m) (TOSD) | yes | yes |  |  |  |  |  |  |  |  |  |  |  |  |
| Height of waves (m) (HOWV) | yes (not in subtype 002) |  | yes | yes | YES | yes |  |  |  |  |  |  |  |  |
| Period of waves (sec) (POWV) | yes (not in subtype 002) |  | yes | yes | YES | yes |  |  |  |  |  |  |  |  |
| Height of wind waves (m) (HOWW) | yes (not in subtype 002) |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Period of wind waves (sec) (POWW) | yes (not in subtype 002) |  | YES |  | yes | yes |  |  |  |  |  |  |  |  |
| Direction of motion of ship during last 3-hrs (TDMP) |  |  | YES |  |  |  |  |  |  |  |  |  |  |  |
| Average speed of motion of ship during last 3-hrs (ASMP) |  |  | YES |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic of pressure tendency (CHPT) | YES | YES | YES | YES | YES | YES |  |  |  |  |  |  |  |  |
| 3-hour pressure change (Pa) (3HPC) | YES | YES | YES | YES | YES | YES |  |  |  |  |  |  |  |  |
| 24-hour pressure change (Pa) (24PC) | yes |  | yes |  |  |  |  |  |  |  |  |  |  |  |
| Airframe icing (AFIC) |  |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Height of base of icing (m) (HBOI) |  |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Height of top of icing (m) (HTOI) |  |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Degree of turbulence (DGOT) |  |  |  |  |  |  |  |  | yes | YES | YES (also YES in subtype 001; yes in subtype 007) |  |  |  |
| Height of base of turbulence (m) (HBOT) |  |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Height of top of turbulence (m) (HTOT) |  |  |  |  |  |  |  |  | yes | YES |  |  |  |  |
| Phase of aircraft flight (POAF) |  |  |  |  |  |  |  |  |  |  | YES | YES |  | YES |
| Turbulence index (TRBX) |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |
| Aircraft roll angle flag (ROLF) |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |
| Precision of temperature observation (PCAT) (K) |  |  |  |  |  |  |  |  |  |  | YES | yes |  | YES |
| Turbulence index for period from 1 min prior to obs time to obs time(TRBX10) |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |
| Turbulence index for period from 2 min prior to obs time to 1 min prior to obs time (TRBX21) |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |
| Turbulence index for period from 3 min prior to obs time to 2 min prior to obs time (TRBX32) |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |
| Turbulence index for period from 4 min prior to obs time to 3 min prior to obs time (TRBX43) |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |

Key:

Italicized variable name – This type will be added to the next PREPBUFR file update.

Italicized yes – This type is in the BUFR table for the particular BUFR report type/subtype, but a listing over several days shows that it is always missing. A red italicized yes means no attempt will be made to encode this type into the PREPBUFR file.

Upper case YES – This type is non-missing for most reports in the particular BUFR report type/subtype.

Lower case yes – This type is non-missing in < 50% of the reports in the particular BUFR report type/subtype.  For type TP01 under report type SHIP – even though there was a rare report with a non-missing value, this type will not be encoded into the PREPBUFR file (yes is red – only case of a non-italicized red yes).

Shaded cells bordered by curly-cue lines - These types are combined in a delayed replication sequence (i.e., there can be more than one “level” of these types in a single report).

1 –	For METAR reports in 000/007, .DTHMXTM/MXTM and .DTHMITM/MITM usually present only around 00, 06, 12, 18Z with one replication over 6-hours at 12 and 18Z and two replications over 6- and 24-hours at 00 and 06Z.

2 –	Appears to not be present for PIBALS in 002/005 and 002/009.

3 –	If SST1 is missing in dump of drifting buoy reports, first level of STMP (sea temperature at specified depth) is used instead if STMP is non-missing in dump and DBSS is no greater than 10 meters (STMP is stored under mnemonic SST1 in PREPBUFR file in this case, DBSS is also stored).

4 –	TPHR (time period in hours) is actually encoded in dump of MESONET reports, but is changed to .DTHTOPC for storage in PREPBUFR file.  Normally there are two replications of .DTHTOPC/TOPC for MESONET reports, the first for total precip. over 1-hour and the  second for total precip. over 24-hours.

5 –	TOPC is defined as water equivalent of ground snow for METAR reports so it is not encoded in the PREPBUFR file.

6 –	If both HBLCS and HOCB are present, use HOCB as it more accurately defines the height of the cloud base.

7 –	Prior to 5/2002, HOCB was present and HBLCS was missing.  After this date, HBLCS replaced HOCB.

Note: Ozone (OZON) was also requested.  It is already in the checkout airnow PREPBUFR files.  It is not found in any other data types that we currently dump and encode into PREPBUFR.

The following requested sensible element types are not found in any data type we currently dump and encode into PREPBUFR:

Precipitation type (mnemonics PRTP, WXTS, WXTP, WXTZ, WXTR) (mostly likely an available entry in present or past weather in mnemonics PRWE, PSW1 and PSW2).

Wind speed at 10 or 5 meters (WS10/WS10H, WS05), wind direction at 10 or 5 meters (WD10/WD10H, WD05).

Probability of sky being clear, scattered, broken, overcast.

Probability of: LIFR, IFR, MVFR.